Glenny Wood Lodge

The Leader in charge to review the risk assessment and add any further requirements specific to meet the needs of their section / group.

| What hazard have you identified? | What are the risks from it? | Who is at risk? | How are the risks already controlled? What extra controls are needed? | What has changed that needs to be thought about and controlled? |
| --- | --- | --- | --- | --- |
| Exiting from the lodge | Fire / Gas leak | All Users | The double doors in the kitchen, at least one must be open (unlocked and unbolted) and the front door unlocked at all times the lodge is in use. |  |
| Gas Cooker | Burns and Gas Leak | All users | Take extra care when lighting the rings and oven, extra-long matches are recommended. When the cooker is not in use ensure all gas rings and the oven is turned off. When the cooker is in use ensure only the rings / oven that is lit is turned on |  |
| Gas leak (LPG – Liquid Petroleum Gas) - risk of gas escaping from the appliances / pipes / connections | Gas leak could cause explosions, nausea, sickness or unconsciousness | Hall users | If you smell gas turn off the oven if working, evacuate the hall and move all users to the fire meeting point beyond the toilet area. Turn off the main supply at the gas stopcock which is situated outside on the kitchen wall, at the far end to the kitchen door entrance. Open the two doors, front and back and open any windows you can to ventilate the hall. Notify the Glenny Team on – 01275562102 |  |
| Central Heating boiler not working and no heating in the hall | In the Winter months hall very cold | Hall users | If you smell gas – as above. Use the push button ad-hoc timer to switch on heating. Positioned on the wall on RHS of serving hatch. If heating still not working notify the Glenny team on – 01275562102 |  |
| Central Heating boiler not working and no hot water from the left hand sink tap | Cause could be boiler failure | Hall users | If you smell gas – as above. Leave the hot tap run for 2 minutes, if no hot water, switch off and try again. If hot water still not working notify the Glenny team on – 01275562102 |  |
| Severe Winds – could cause Trees to fall or branches to break | Severe injury / death due to falling trees / branches | Site users | Professional Tree survey carried out annually and recommendations implemented as necessary. Monthly tree walk inspection undertaken by Glenny team member. Be aware of potential hazards when camping near trees Winds at 32 mph or above consideration must be given by site user to leave the site for safety reasons. See Woodland Management – Windy conditions statement – https://glennywood.org.uk/woodland.html |  |
| Excess rain/muddy slippery floors | Cuts, sprains, breaks | Site users | Users to remove boots before entering the lodge Brief on dangers of slipping in muddy conditions. Remind users of slippery conditions on the lodge patio. Apply First Aid as appropriate. |  |
| Uneven terrain through the campsite, woods & paths when leaving the lodge including walking to the loos | Physical injury from trips, falls, etc. | Site users | Brief on dangers of tripping/falling as necessary. Remember Bluebell plants are very slippery so don’t walk on them. In the dark use lights when walking to car park and loos. Buttons by both front and side doors. Give First Aid as required |  |
| Unseen logs / tree trunks or other obstruction on the ground covered by bracken / other cover. | Physical injury from trips, falls, etc. | Site users | Make those in your party aware of potential hidden hazards. Brief on dangers of tripping/falling as necessary. Give First Aid as required |  |
| Using Glenny Wood Site | Physical harm | Users | Use the Glenny Wood Campsite Risk Assessment form and undertake your own review and brief users. |  |
| Using site Activities | Physical harm | Users | Use the Glenny Wood Individual Risk Assessment forms and undertake your own review of each activity and brief users. |  |
| Wetlands | Drowning | Site users | Leader to make this area out of bounds When in use supervision at all times See separate draft Risk Assessment |  |